FRAMING NOTES

1.  All exterior wall openings and bearing wall openings to have 4x12 headers unless otherwise indicated.

2.  Joists that are attached to flush beams are to be hung with “Simpson” U-210 or equivalent.

3.  Provide double joists under all parallel partitions over provided fireblocking, draftstops and firestops as per

the U.B.C. Section   2516F.

4.  Lumber Species:

A.  Post, beams, headers, joists and rafters				No. 2 Douglas fir

B.  Sills. Plates, blocking, bridging, etc.				No. 3 Douglas fir.

C.  Studs								Stud grade D.F.

D.  Post and beam decking						Utility grade D.F.

E.  Plywood sheathing							\”CDX plywood, 32/16

F.  Glu-Lam beams							Fb-2400, dry adh.

5.  Nail Schedule:

Joist to sill or girder			3-8d				Toe nail

Bridging to joist			2-8d				Toe nail

2” Subfloor to girder			2-16d				Blind nail

Sole plate to joist			16d @ 16”			Face nail

Top plate to studs			2-16d				End nail

Stud to sole plate			4-8d				Toe nail or End nail

Double studs				16d @ 16”			Face nail

Double top plate			16d @ 16”			Face nail

Continuous header (2 pc.)		16d @ 16”			Edge nail

Clg. Joist to pl.			3-8d				Toe nail

Clg. Joist lap over pl.			3-16d				Face nail

Clg. Joist to rafter			3-16d				Face nail

Rafter to top pl.			3-8d				Toe nail

Build-up corner studs			16d @ 24”			Face nail

Plywood subfloor			8d @ 6”			Edge nail

8d @ 10”			Interior

Plywood & roof sheathing		8d @ 6”			Edge nail

8d @ 12”			Interior

Top pl. at intersections		2-16d				Face nail

Multiple joists (up to 3)		16d @ 15”			Stagger nail

Multiple joists (over 3)		\”~Bolts w/washers

Each side @ 24” O.C.

1X6 Spaced sheathing			2-8d				Face nail